សាកលវិទ្យាល័យភូមិន្ទភ្នំពេញ

Royal University of Phnom Penh

មហាវិទ្យាល័យវិស្វកម្ម

Faculty of Engineering

ដេប៉ាតឺម៉ង់ វិស្វកម្មទូរគមនាគមន៍និង​អេឡិចត្រូនិច

Department of Telecommunication and Electronic Engineering

ESP32

អៀន​ ស្រីនិច

Ean sreynich

January 2023

Table of Contents

- OVERVIEW......................................................................................

  - ESP32 Features and Specification...............................................................

- ESP32 Packaged chip and module..................................................................

  - ESP32 Chip...................................................................................................

  - ESP Module ………....................................................................................

Reference

List of figures

Figure 1. ESP32 NODEMCU.............................................................................................................

Figure 2. ESP-WROOM-32...............................................................................................................

- OVERVIEW

ESP32 is a series of low-cost, low-power system on a chip microcontroller with integrated WIFI and dual-mode Bluetooth. ESP32 is created and developed by Espressif Systems, a Shanghai-based Chinese company, and is manufactured by TSMC using their 40 nm process. It is a successor to the ESP8266 microcontroller.

Figure 1. ESP NODEMCU

  - ESP32 Features and Specification

Power supply of the ESP32

- Power supply: 2.3V – 3.6V

- Current consumption: 20uA – 240mA. In deep sleep mode less than 5uA

- Operating temperature rang: -40c – 125c

Interface

- 34 × programmable GPIOs

- 12-bit SAR ADC up to 18 channels

- 2 × 8-bit DACs

- 10 × touch sensors (capacitive sensing GPIOs)

- SD/SDIO/CE-ATA/MMC/eMMC host controller

- SDIO/SPI slave controller

- Ethernet MAC interface with dedicated DMA and planned IEEE 1588 Precision Time Protocol support[4]

- Infrared remote controller (TX/RX, up to 8 channels)

- Pulse counter (capable of full quadrature decoding)

- Motor PWM

- LED PWM (up to 16 channels)

- Hall effect sensor

- Ultralow power analog pre-amplifier

Network

- WiFi 802.11 b/g/n 2.4 GHz with WPA/WPA2 PSK

- Ipv4 and ipv6 from Arduino Core 2.5.o

- Bandwidth: 150 to 300kBytes/s

- Bluetooth: Version 4.2

CPU: 160MHz up to 240MHz

Internal Memory

- 448 Kbytes ROM for booting

- 520 Kbytes on-chip SRAM for data and instruction

- 8 Kbytes SRAM in RTC

- ESP32 Packaged chip and module

2.1 ESP chips

The ESP32 system on a chip integrated circuit is packaged in both 6 mm × 6 mm and 5 mm × 5 mm sized QFN packages.

Table 1: ESP chip

| Identifier | Processor cores | Embedded flash memory (MiB) | Package size | Description |
| --- | --- | --- | --- | --- |
| ESP31B | 2 | 0 | 6 mm×6 mm | Pre-release SoC used for beta testing; no longer available. |
| ESP32-D0WDQ6 | 2 | 0 |  | Initial production release chip of the ESP32 series. |
| ESP32-D0WD | 2 | 0 | 5 mm×5 mm | Smaller physical package variation similar to ESP32-D0WDQ6. |
| ESP32-D2WD | 2 | 2 |  | 2 MiB (16 Mibit) embedded flash memory variation. |
| ESP32-U4WDH | 1 | 4 |  | Single-core processor and 4 MiB (32 Mibit) embedded flash memory variation. |
| ESP32-S0WD | 1 | 0 |  | Single-core processor variation. |

2.2 ESP32 Module

The pins function of the ESP32 is explained in detail in the article on the ESP32 pinout

Figure 2. ESP-WROO

Reference

https://www.espressif.com/en/products/modules

https://en.wikipedia.org/wiki/ESP32